UART Receiver module working Principle.

The UART receiver module converts incoming serial data on the RX line into parallel and asserts a ‘completed’ signal after each byte (series of 8-bits) is processed.

Input: RX data (in series)

Output: Converted data (in parallel) and ‘completed’ signal after conversion.

Serial to parallel converter with (completion done after each byte is converted).

How the RX module converts Series to Parallel

Acknowledges the start bit (falling edge of the signal) waits half of the period of the signal T/2 to align with the bit centre.

Individual bits are sampled once every period T,

2 Ts are allowed to acknowledge stop bits and reset is done.

T = 1 / baud rate

Baud rate is the number of symbols transmitted per second (like bandwidth).

The stop bit should be ‘1’ signifying the end of the data frame sent on the rx_line. The frame error is set to 1 if the stop bit is 0 instead of 1 and back to 0 upon reset.

Testbench of the UART RX protocol

The testbench is simply a way of checking the behaviour of our digital circuit without putting it on our FPGA. We have the opportunity to do so by passing signals to the testbench and observing its behaviour. No need to pass Input/output to the testbench.

The four building blocks

1. The timescale — this is your # unit

verilog

`timescale 1ns/1ps

This means #1 = 1 nanosecond, and simulation resolves to 1 picosecond precision. So for a 100 MHz clock, half a period is #5 (5 ns). This goes at the very top, before the module declaration.

2. The module declaration — no ports

verilog

module UART_RX_tb;

// no inputs, no outputs

endmodule

3. Signal declarations — mirrors the DUT's ports

You declare reg for everything you're driving into the DUT, and wire for everything coming out of the DUT:

verilog

// Driven BY the testbench INTO the DUT → reg

reg        clk   = 0;

reg        rx    = 1;    // idle high

reg        reset = 0;

// Driven BY the DUT OUT to the testbench → wire

wire [7:0] result;

wire       completed;

wire       frame_error;

The rule is simple — if the testbench is the one assigning it, it's a reg. If the DUT is driving it, it's a wire.

4. DUT instantiation — this is where #(...) parameters go

verilog

UART_RX #(

.CLK_FREQ(100_000_000),

.BAUD_RATE(9600)

) dut (

.clk        (clk),

.rx         (rx),

.reset      (reset),

.result     (result),

.completed  (completed),

.frame_error(frame_error)

);

The #(.CLK_FREQ(...), .BAUD_RATE(...)) block overrides the module parameters — this is where simulation gets expensive at real baud rates. At 9600 baud with a 100 MHz clock, one bit period is 10,416 clock cycles, so sending one byte takes ~83,000 clock cycles. That's fine for simulation but slow to write stimulus for manually, which is why testbenches use tasks.

`timescale                   ← sets what # means

module tb;                  ← no ports (No input/output)

reg/wire declarations      ← mirrors DUT ports

DUT instantiation          ← connects signals

clock generation           ← always block toggling clk

task definitions (sub routine)           ← reusable stimulus (send_byte etc.)

initial block              ← the actual test sequence this is where the test runs through a series of tasks.

endmodule ← closes the simulation